# Module Name: Reg_Access

Unit Version: 3

## 1. Feature

The Reg_Access module serves as the primary control and datapath wrapper for processing Register Access commands within the UCIe PHY Sideband (PHY_SB) layer. It decodes incoming 128-bit sideband packets (reg_msg), coordinates with the internal Register File (Reg_File) to execute 32-bit/64-bit Configuration (CFG) or Memory (MEM) Reads and Writes, and safely packages response packets (completion_msg) using even parity schemes.

A critical feature implemented in this RTL version is the UR/CA Fast-Forward Execution: any transaction flagged with a Parity Error, False Destination ID, or Poison bit bypasses the Register File execution entirely and immediately generates an Unsupported Request (UR) completion carrying the original request header as payload.

## 2. Architecture & Operations

The module decomposes the processing pipeline into three tightly-coupled sub-modules:

### A. Reg_DePacketizer (Parsing Stage)

- Packet Capture: On a valid handshake (reg_vld && reg_rdy), the full 128-bit packet is registered into latched_pkt.

- Field Extraction: The lower 64 bits are mapped into the sb_header_u union to split fields like opcode, tag, be, and dstid. The upper 64 bits (latched_pkt[127:64]) are routed straight out as rf_wdata.

- Address Decoding: Extends the header's 24-bit address (hdr.req.addr) into a 25-bit internal address (rf_addr). Bit [24] is decoded as the space bit (asserted 0 for CFG opcodes, and 1 for MEM/DMS opcodes).

- Pre-Execution Error Checks: * Compares the received control parity bit (hdr.req.cp) against the calculated even parity over the lower 62 bits.

  - Asserts false_msg if the packet's destination ID (dstid) does not match LOCAL_PHY (3'b010).

### B. Reg_Access_FSM (Control & Sequencing Stage)

The controller tracks a 4-state synchronous state machine (IDLE, DECODE, EXECUTE, GEN):

- IDLE (2'b00): Asserts reg_rdy. When reg_vld is sensed, transitions to DECODE.

- DECODE (2'b01): Evaluates compilation errors (error = parity_err || false_msg || ep || rf_addr_err). If any error flag is high, it branches directly to GEN to block malicious or corrupted inputs from hitting the hardware registers. Otherwise, it moves to EXECUTE.

- EXECUTE (2'b10): Asserts rd_en (for reads) or wr_en (for writes). For write operations, it exits instantly to GEN. For reads, it stalls until the register file drives rdata_vld high.

- GEN (2'b11): Strops completion_start high for 1 cycle. Sets status = 3'b001 (UR) if an error occurred, or 3'b000 (SC) for successful completion. It loops back to IDLE once downstream clears completion_rdy.

### C. Completion_gen (Packet Construction Stage)

- Opcode Enforcement: Maps out response codes based on transaction success and sizing. Successful reads assign SB_COMPLETION_WITH_32_DATA or SB_COMPLETION_WITH_64_DATA. Successful writes trigger SB_COMPLETION_WITHOUT_DATA.

- Payload Muxing: Muxes the data returning to the network. For valid reads, it formats rf_rdata. For aborted transactions (status != 0), it automatically reflects the Original_Header back to the requester.

- Even Parity Generation: Computes Data Parity (dp) via a continuous XOR tree (^cpl_payload) over the payload. In the sequential block, it handles Control Parity (cp_final) by calculating the XOR reduction over the final 63 header bits (^hdr_low) before outputting the 128-bit transaction to the link.

## 3. Interfaces Signals List

### Table 3.1: Top-Level Reg_Access Interface

| Signal Name | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | Input | 1 | System clock source for the sideband interface. |
| rst_n | Input | 1 | Active-low asynchronous reset. |
| reg_msg[127:0] | Input | 128 | Incoming raw 128-bit Register Access sideband request packet. |
| reg_vld | Input | 1 | Valid push signal from the RDI_control router. |
| reg_rdy | Output | 1 | Backpressure handshake signal indicating module is ready. |
| completion_msg[127:0] | Output | 128 | Formatted 128-bit sideband response message. |
| completion_vld | Output | 1 | Valid signal to Link TX Arbiter. |
| completion_rdy | Input | 1 | Clear-to-send handshake indicator from the TX path. |
| rf_addr[24:0] | Output | 25 | Fully formed register byte address {space, addr[23:0]}. |
| rf_be[7:0] | Output | 8 | Active-byte write enables derived from the request header. |
| rf_is_64b_access | Output | 1 | Status bit indicating a 64-bit transaction length when high. |
| rf_wdata[63:0] | Output | 64 | 64-bit write data payload extracted from reg_msg[127:64]. |
| rd_en | Output | 1 | Active-high read prompt to the local Register File. |
| wr_en | Output | 1 | Active-high write prompt to the local Register File. |
| rf_rdata[63:0] | Input | 64 | Read data payload retrieved from the Register File. |
| rdata_vld | Input | 1 | Done status indicating valid read data is sitting on rf_rdata. |
| addr_err_o | Input | 1 | Address decode fault signal sourced from the Register File. |

Table 3.2: Reg_DePacketizer Sub-module Interface

| Signal Name | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | Input | 1 | Clock input. |
| rst_n | Input | 1 | Asynchronous reset. |
| reg_msg[127:0] | Input | 128 | 128-bit raw request packet input. |
| reg_vld | Input | 1 | Valid incoming packet indicator. |
| reg_rdy | Input | 1 | Intercepted ready handshake signal used to latch the input. |
| opcode[4:0] | Output | 5 | Decoded transaction type code (sb_opcode_e). |
| parity_err | Output | 1 | High if the evaluated even parity check detects a corruption. |
| ep | Output | 1 | Error Poison flag extracted directly from request header bit [5]. |
| false_msg | Output | 1 | High if dstid target is illegal (not matching LOCAL_PHY). |
| rf_addr | Output | 25 | Synthesized address layout for the Register File. |
| rf_be | Output | 8 | Extracted byte enable array. |
| rf_is_64b_access | Output | 1 | High if opcode maps to a 64-bit transaction. |
| rf_wdata | Output | 64 | Write payload stripped from packet upper half. |
| Original_Header | Output | 64 | Preserved 64-bit transaction header to pass to Completion_gen. |

Table 3.3: Reg_Access_FSM Sub-module Interface

| Signal Name | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | Input | 1 | Clock input. |
| rst_n | Input | 1 | Asynchronous reset. |
| reg_vld | Input | 1 | Input transaction request valid signal. |
| reg_rdy | Output | 1 | Handshake state output (Asserted high in IDLE). |
| completion_rdy | Input | 1 | Readiness signal check from downstream TX logic. |
| opcode | Input | 5 | Current request opcode from parsing block. |
| parity_err | Input | 1 | Pre-calculated parity mismatch error line. |
| ep | Input | 1 | Poison bit indicator from packet. |
| false_msg | Input | 1 | Invalid target routing error flag. |
| rd_en | Output | 1 | Active-high read strobe out to Register File. |
| wr_en | Output | 1 | Active-high write strobe out to Register File. |
| rdata_vld | Input | 1 | Valid read data gate line from Register File. |
| rf_addr_err | Input | 1 | Target fault error feedback from Register File. |
| status | Output | 3 | Outbound response status code (3'b000 = Success, 3'b001 = UR). |
| completion_start | Output | 1 | One-cycle execution pulse triggering packet assembly. |

Table 3.4: Completion_gen Sub-module Interface

| Signal Name | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | Input | 1 | Clock input. |
| rst_n | Input | 1 | Asynchronous reset. |
| completion_start | Input | 1 | FSM sequence generation trigger. |
| status | Input | 3 | FSM status block input routing. |
| Original_Header | Input | 64 | Cached header image preserved during parsing stage. |
| rf_rdata | Input | 64 | Active datapath line carrying register contents. |
| rdata_vld | Input | 1 | Read content flag from register block. |
| completion_msg | Output | 128 | Fully calculated 128-bit sideband output completion packet. |
| completion_vld | Output | 1 | Active-high output valid transaction line. |
| completion_rdy | Input | 1 | Backpressure line checking downstream buffer space. |