Training_Mgmt

Block Documentation  |  Unit Version: latest

# 1.  Feature

This block acts as the intermediate layer between the Protocol/Training logic (RDI_SM, LTSM) and the Physical transmission layer (Link Controller / SerDes). Its main features include:

| Feature | Description |
| --- | --- |
| Packetization / De-packetization | Converts RDI_SM and LTSM coded control signals into standard 128-bit packets for transmission, and decodes received packets back to their respective destinations. |
| Protocol Isolation | Dedicated encoder/decoder pairs (RDI_Packetizer, LTSM_Packetizer, RDI_De-Packetizer, LTSM_De-Packetizer) ensure each protocol block receives only its relevant data with no cross-contamination. |
| Buffering | Two Synchronous FIFOs (RDI_MSG_FIFO and LTSM_MSG_FIFO, each 128-bit wide) absorb simultaneous messages from both sources and guarantee no data loss. |
| Arbitration | A Round-Robin Arbiter fairly schedules outgoing packets from the two FIFOs onto the shared Link Controller channel. |
| Flow Control | Manages handshake signals — Link_ready from the Link Controller (TX) and push_ready back to the Packetizers — to ensure data integrity throughout transmission. |

# 2.  Architecture & Operations

The Training_Mgmt block is internally divided into two independent data paths: the Transmit (TX) path and the Receive (RX) path. Each path has its own dedicated set of sub-blocks to ensure clean protocol isolation and predictable dataflow.

2.1  TX Path  (Training_Mgmt → Link Controller)

The TX path is responsible for encoding & packaging outgoing messages from the RDI_SM and LTSM into 128-bit standard packets and forwarding them to the Link Controller in a fair, flow-controlled manner.

### RDI_MSG_FIFO  [DEPTH-1:0][127:0]

• Receives raw RDI transmit requests from RDI_SM.

• Buffers the RDI message opcode together with the stall indication.

• Exposes valid/ready handshake to the Arbiter.

• Does not perform packet formatting.

### LTSM_MSG_FIFO  [DEPTH-1:0][127:0]

• Receives raw LTSM message fields.

• Buffers.

- Message code.

- Message Info.

- Payload.

• Exposes valid/ready handshake to the Arbiter..

### Arbiter  (Round-Robin)

• Logic: Implements a Round-Robin algorithm, toggling between RDI_MSG_FIFO and LTSM_MSG_FIFO based on the not_empty status flags.

• Operation: Pops the selected message entry from either FIFO and forwards the corresponding message fields to the Packetizer.

• Handshake: Holds data and valid high until Link_ready is asserted by the Link Controller, then de-asserts valid and advances to the next cycle.

• Backpressure: Asserts push_ready toward both Packetizers to signal FIFO availability.

### Packetizer

•  Receives the arbitrated message fields from the Round-Robin Arbiter..

• Builds the standard 128-bit Training Packet.

• Inserts the proper packet header based on the message type.

• Generates trn_msg_send and trn_vld_send.

• Holds the packet until trn_rdy is asserted.

2.2  RX Path  (Link Controller → Training_Mgmt)

The RX path receives raw 128-bit packets from the Link Controller and routes the decoded payloads to the appropriate destination block (RDI_SM or LTSM).

### De-Packetizer

•  Receives trn_msg_rcvd.

•  Decodes the 128-bit Training Packet.

• Extracts.

- Message code.

- Message Info.

- Payload.

- Stall bit.

•  Outputs decoded fields together with a valid indication.

### Training_Mgmt_Demux

• Receives decoded fields from the DePacketizer.

• Determines the destination protocol based on the decoded Message Number.

• Routes RDI messages to RDI_SM.

• Routes LTSM messages to LTSM.

• Generates the corresponding valid signal for the selected destination.

## 1. Top Module: Training_Mgmt

| Signal Name | Direction | Width | Description / Interface Domain |
| --- | --- | --- | --- |
| clk_main | Input | 1-bit | Main State Machine domain clock (LTSM/RDI_SM) |
| rst_main_n | Input | 1-bit | Main SM domain active-low reset |
| clk_sb | Input | 1-bit | Sideband domain clock |
| rst_sb_n | Input | 1-bit | Sideband domain active-low reset |
| RDI_msg_no_send | Input | [7:0] | RDI Message Number to be transmitted |
| stall_send | Input | 1-bit | Stall command from Main controller |
| RDI_vld_send | Input | 1-bit | Valid flag for outgoing RDI message |
| RDI_rdy | Output | 1-bit | Ready signal to RDI transmitter interface |
| ltsm_msg_n_send | Input | [7:0] | LTSM Message Number to be transmitted |
| msg_data_send | Input | [63:0] | LTSM Message payload data |
| msg_info_send | Input | [15:0] | LTSM Message additional metadata/info |
| ltsm_vld_send | Input | 1-bit | Valid flag for outgoing LTSM message |
| ltsm_rdy | Output | 1-bit | Ready signal to LTSM transmitter interface |
| RDI_vld_rcvd | Output | 1-bit | Valid flag for demuxed incoming RDI message |
| RDI_msg_no_rcvd | Output | [7:0] | Decoded incoming RDI message number |
| stall_rcvd | Output | 1-bit | Decoded incoming stall command |
| ltsm_vld_rcvd | Output | 1-bit | Valid flag for demuxed incoming LTSM message |
| ltsm_msg_no_rcvd | Output | [7:0] | Decoded incoming LTSM message number |
| msg_data_rcvd | Output | [63:0] | Decoded incoming LTSM payload data |
| msg_info_rcvd | Output | [15:0] | Decoded incoming LTSM message info |
| trn_msg_send | Output | [127:0] | 128-bit packetized message sent to external Link Controller |
| trn_vld_send | Output | 1-bit | Valid handshake for transmitted link message |
| trn_rdy | Input | 1-bit | Backpressure/ready signal from Link Controller |
| trn_msg_rcvd | Input | [127:0] | 128-bit raw message received from external Link Controller |
| trn_vld_rcvd | Input | 1-bit | Valid handshake for received link message |

## 2. Internal Sub-Modules

### 2.1 DePacketizer Module

| Signal Name | Direction | Width | Connected To / Description |
| --- | --- | --- | --- |
| clk | Input | 1-bit | System clock |
| rst_n | Input | 1-bit | Active-low reset |
| msg_in | Input | [127:0] | Raw packetized message from top input |
| vld_in | Input | 1-bit | Valid flag for msg_in |
| msg_no_out | Output | msg_no_e | Decoded message type enumeration |
| msginfo_r | Output | [15:0] | Decoded message info metadata |
| payload_r | Output | [63:0] | Decoded 64-bit message payload |
| vld_r | Output | 1-bit | Valid signal for the completely decoded fields |
| stall_rcvd | Output | 1-bit | Decoded stall status flag |

### 2.2 Packetizer Module

| Signal Name | Direction | Width | Connected To / Description |
| --- | --- | --- | --- |
| clk | Input | 1-bit | System clock |
| rst_n | Input | 1-bit | Active-low reset |
| msg_data_send | Input | [63:0] | Payload data coming from the Arbiter block |
| msg_info_send | Input | [15:0] | Metadata info from the Arbiter block |
| msg_no_send | Input | [7:0] | Message code from the Arbiter block |
| valid_send | Input | 1-bit | Valid status signal from the Arbiter block |
| trn_rdy | Input | 1-bit | External ready handshake from the Link Controller |
| stall_send | Input | 1-bit | Special RDI stall flag bit to packetize |
| trn_msg_send | Output | [127:0] | Final formatted 128-bit link message packet |
| trn_vld_send | Output | 1-bit | Handshake valid flag for the 128-bit link message |
| rdy | Output | 1-bit | Ready flow-control feedback to the Arbiter block |

### 2.3 Training_Mgmt_Demux Module

| Signal Name | Direction | Width | Connected To / Description |
| --- | --- | --- | --- |
| rx_msg_no_raw | Input | msg_no_e | Decoded message enumeration from DePacketizer |
| rx_msginfo_raw | Input | [15:0] | Decoded message metadata info from DePacketizer |
| rx_payload_raw | Input | [63:0] | Decoded message payload from DePacketizer |
| rx_vld_raw | Input | 1-bit | Decoded valid flag from DePacketizer |
| rx_stall_raw | Input | 1-bit | Decoded stall flag from DePacketizer |
| RDI_vld_rcvd | Output | 1-bit | Demultiplexed valid flag for the RDI client interface |
| RDI_msg_no_rcvd | Output | [7:0] | Demultiplexed RDI message identification code |
| stall_rcvd | Output | 1-bit | Demultiplexed active stall command indicator |
| ltsm_vld_rcvd | Output | 1-bit | Demultiplexed valid flag for LTSM client interface |
| ltsm_msg_no_rcvd | Output | [7:0] | Demultiplexed LTSM message identification code |
| msg_data_rcvd | Output | [63:0] | Demultiplexed 64-bit LTSM message payload data |
| msg_info_rcvd | Output | [15:0] | Demultiplexed 16-bit LTSM message metadata info |

### 2.4 Round-Robin Arbiter (RR_arbiter) Module

| Signal Name | Direction | Width | Connected To / Description |
| --- | --- | --- | --- |
| clk | Input | 1-bit | System clock |
| rst_n | Input | 1-bit | Active-low reset |
| ltsm_valid | Input | 1-bit | Valid data ready signal from LTSM FIFO module |
| ltsm_data | Input | [DATA_WIDTH-1:0] | Data payload coming directly out of LTSM FIFO |
| ltsm_ready | Output | 1-bit | Pop read-enable acknowledgement to LTSM FIFO |
| rdi_valid | Input | 1-bit | Valid data ready signal from RDI FIFO module |
| rdi_data | Input | [DATA_WIDTH-1:0] | Data payload coming directly out of RDI FIFO |
| rdi_ready | Output | 1-bit | Pop read-enable acknowledgement to RDI FIFO |
| link_ready | Input | 1-bit | Ready signal from down-stream Packetizer block |
| link_valid | Output | 1-bit | Arbitrated valid grant flag given to Packetizer |
| link_data | Output | [DATA_WIDTH-1:0] | The selected winning data stream passed to Packetizer |

### 2.5 Parameterized FIFO Module (fifo) عاوزة اضيف بلوك ليها لوحده

| Signal Name | Direction | Width | Connected To / Description |
| --- | --- | --- | --- |
| W_CLK | Input | 1-bit | Write-side domain clock |
| WRST_N | Input | 1-bit | Write-side domain active-low asynchronous reset |
| WINC | Input | 1-bit | Write push increment enable |
| WR_DATA | Input | [DATA_WIDTH-1:0] | Input data vector (88 bits for LTSM, 9 bits for RDI) |
| WFULL | Output | 1-bit | Synchronized full flag status indicator |
| WREADY | Output | 1-bit | Inverted full flag indicating ready to receive write |
| R_CLK | Input | 1-bit | Read-side domain clock (connected to clk_sb) |
| RRST_N | Input | 1-bit | Read-side domain active-low asynchronous reset |
| RINC | Input | 1-bit | Read pop increment enable from arbiter grant |
| RD_DATA | Output | [DATA_WIDTH-1:0] | Output data vector sent directly to arbiter inputs |
| REMPTY | Output | 1-bit | Synchronized empty flag status indicator |
| RVALID | Output | 1-bit | Inverted empty flag indicating valid data is available |